The following JSON template provides a production-ready Jupyter Notebook structure configured for statistical quality control, distribution analysis, machine capability assessment, and bivariate regression.

## Jupyter Notebook Template (statistical_analysis_template.ipynb)

JSON

{ "cells": [  {   "cell_type": "markdown",   "metadata": {},   "source": [    "# Statistical Analysis & Quality Control Pipeline\n",    "**Objective:** Process numerical datasets, evaluate location/dispersion parameters, assess capability indices, and generate control charts."   ]  },  {   "cell_type": "code",   "execution_count": null,   "metadata": {},   "outputs": [],   "source": [    "# Phase 1: Environment Setup & Data Ingestion\n",    "import numpy as np\n",    "import scipy.stats as sta\n",    "from scipy.integrate import quad\n",    "import matplotlib.pyplot as plt\n",    "import time as t\n",    "\n",    "# Load dataset from text file\n",    "data_path = 'data.txt'\n",    "values = np.loadtxt(data_path)  # Load sequence\n",    "n = len(values)\n",    "\n",    "# Reshape into sample matrix (5 rows per batch) if running SPC\n",    "rows, columns = 5, n // 5\n",    "sample_table = np.reshape(values[:rows*columns], (rows, columns), order='F')  # Fortran order column formatting\n",    "print(f\"Loaded {n} records into a matrix of shape {sample_table.shape}\")"   ]  },  {   "cell_type": "code",   "execution_count": null,   "metadata": {},   "outputs": [],   "source": [    "# Phase 2: Location & Dispersion Parameter Evaluation\n",    "mean_val = np.mean(values)  # Arithmetic mean\n",    "median_val = np.median(values)  # Median\n",    "mode_res = sta.mode(values, axis=None)  # Modal value\n",    "hmean_val = sta.hmean(np.abs(values))  # Harmonic mean\n",    "gmean_val = sta.gmean(np.abs(values))  # Geometric mean\n",    "std_val = np.std(values, ddof=1)  # Sample standard deviation (n-1)\n",    "skew_val = sta.skew(values)  # Exact distribution skewness\n",    "\n",    "print(f\"Mean: {mean_val:.4f} | Median: {median_val:.4f} | Mode: {mode_res.mode}\")\n",    "print(f\"Std Dev: {std_val:.4f} | Skewness: {skew_val:.4f} ({'Left' if skew_val > 0 else 'Right'}-skewed)\")"   ]  },  {   "cell_type": "code",   "execution_count": null,   "metadata": {},   "outputs": [],   "source": [    "# Phase 3: Machine Capability Analysis (Cm & Cmk)\n",    "setpoint = 50.0  # Target setpoint value\n",    "To, Tu = 5.0, -5.0  # Upper/Lower tolerance specification limits\n",    "T = To - Tu  # Total tolerance range\n",    "\n",    "Cm = T / (6 * std_val)  # Machine capability index[cite: 1]\n",    "UCL_spec, LCL_spec = setpoint + To, setpoint + Tu  # Tolerance limits[cite: 1]\n",    "delta_k = min(UCL_spec - mean_val, mean_val - LCL_spec)  # Critical deviation distance[cite: 1]\n",    "Cmk = delta_k / (3 * std_val)  # Machine capability characteristic value[cite: 1]\n",    "\n",    "print(f\"Capability Index (Cm): {Cm:.4f}\")\n",    "print(f\"Critical Capability (Cmk): {Cmk:.4f} (Status: {'PASS' if Cmk >= 1.67 else 'FAIL'})\")[cite: 1]"   ]  },  {   "cell_type": "code",   "execution_count": null,   "metadata": {},   "outputs": [],   "source": [    "# Phase 4: Quality Control Chart Simulation (x-bar & s Charts)\n",    "A3, B4 = 1.152, 1.669  # Factors for sample size n=5[cite: 1]\n",    "mw = [np.mean(sample_table[:, i]) for i in range(columns)]  # Batch means[cite: 1]\n",    "staw = [np.std(sample_table[:, i], ddof=1) for i in range(columns)]  # Batch std dev[cite: 1]\n",    "\n",    "mmw, mws = np.mean(mw), np.mean(staw)  # Global sample averages[cite: 1]\n",    "UCLm, LCLm = mmw + A3 * mws, mmw - A3 * mws  # Control limits for means[cite: 1]\n",    "UCLs = B4 * mws  # Control limit for standard deviations[cite: 1]\n",    "\n",    "fig, ax = plt.subplots(2, 1, figsize=(8, 6))\n",    "h = np.arange(1, columns + 1)\n",    "ax[0].plot([1, columns], [UCLm, UCLm], 'r--', [1, columns], [mmw, mmw], 'g-', [1, columns], [LCLm, LCLm], 'r--')[cite: 1]\n",    "ax[0].plot(h, mw, 'bx-')[cite: 1]\n",    "ax[0].set_title(\"Mean Value Chart (x-bar)\")[cite: 1]\n",    "\n",    "ax[1].plot([1, columns], [UCLs, UCLs], 'r--')[cite: 1]\n",    "ax[1].plot(h, staw, 'gx-')[cite: 1]\n",    "ax[1].set_title(\"Standard Deviation Chart (s)\")[cite: 1]\n",    "fig.tight_layout()[cite: 1]\n",    "plt.show()"   ]  },  {   "cell_type": "code",   "execution_count": null,   "metadata": {},   "outputs": [],   "source": [    "# Phase 5: Linear Regression Analysis\n",    "# Example input arrays X (independent) and Y (dependent)[cite: 1]\n",    "X = np.array([46, 53, 29, 61, 36, 39, 47, 49, 52, 38, 55, 32, 57, 54, 44])[cite: 1]\n",    "Y = np.array([12, 15, 7, 17, 10, 11, 11, 12, 14, 9, 16, 8, 18, 14, 12])[cite: 1]\n",    "\n",    "m_slope, a_intercept, r_val, p_val, std_err = sta.linregress(X, Y)  # Fit linear model[cite: 1]\n",    "print(f\"Model: y = {m_slope:.4f}x + {a_intercept:.4f} | Correlation r: {r_val:.4f}\")[cite: 1]\n",    "\n",    "plt.figure(figsize=(6, 4))\n",    "plt.plot(X, Y, 'rx', label='Data')[cite: 1]\n",    "plt.plot(X, m_slope * X + a_intercept, 'b-', label='Fit')[cite: 1]\n",    "plt.xlabel(\"Influencing Variable X\")[cite: 1]\n",    "plt.ylabel(\"Outcome Variable Y\")[cite: 1]\n",    "plt.legend()[cite: 1]\n",    "plt.show()"   ]  } ], "metadata": {  "language_info": {   "name": "python"  } }, "nbformat": 4, "nbformat_minor": 2}

## Deployment Instructions

- Save the raw JSON code block above to a file named statistical_analysis_template.ipynb.

- Open Jupyter Lab or VS Code and launch the template file.

- Ensure required dependencies (numpy, scipy, matplotlib) are installed in your active Python environment[cite: 1].

- Place raw input data in data.txt formatted as standard numeric space/newline-delimited text[cite: 1].

- Run the cells sequentially to perform the complete statistical workflow[cite: 1].

## Project Phases & Coding Strategy

1. Data Ingestion & Array Formatting

- Objective: Parse raw operational logs, sensor readings, or sample batches into structured array formats[cite: 1].

- Coding Technique: High-performance file parsing using numpy.loadtxt() and vector reshaping using numpy.reshape(..., order='F') to structure 1D measurement streams into batch matrices[cite: 1].

2. Location & Dispersion Characterization

- Objective: Quantify central tendency, tail asymmetry, and scatter across production metrics[cite: 1].

- Coding Technique: Fast C-compiled vector calculations using numpy.mean(), numpy.median(), numpy.std(ddof=1), alongside specialized distribution estimators from scipy.stats (skew, mode, hmean, gmean)[cite: 1].

3. Process Capability & Risk Evaluation

- Objective: Assess whether process variability meets engineering specifications and quality thresholds[cite: 1].

- Coding Technique: Algorithmic conditional logic evaluating formulaic indices:$$C_m = \frac{T}{6s}$$[cite: 1]$$C_{mk} = \frac{\Delta_{krit}}{3s}$$[cite: 1]Determining compliance against the standard target ratio ($C_{mk} \ge 1.67$)[cite: 1].

4. Quality Control Charting (SPC)

- Objective: Continuous operational monitoring to catch process drift, trends, or limit violations before defect production occurs[cite: 1].

- Coding Technique: Slicing operations table[0:rows, i] combined with matplotlib.pyplot.subplots(2, 1) to display synchronized $\bar{x}$ (mean) and $s$ (standard deviation) polylines relative to control limits ($UCL_{\bar{x}}, LCL_{\bar{x}}, UCL_s$)[cite: 1].

5. Bivariate Modeling & Regression

- Objective: Quantify dependency relationships and correlations between independent environmental variables and target outcomes[cite: 1].

- Coding Technique: Single-pass linear regression fits via scipy.stats.linregress(X, Y), extracting slope ($m$), y-intercept ($a$), correlation coefficient ($r$), and plotting linear scatter overlays using matplotlib[cite: 1].

The R language is natively built for statistical computing, linear algebra, and data visualization. Below is a targeted code cheat sheet covering data reshaping, location/dispersion metrics, process capability calculations, numerical integration, quality control charting, and regression analysis.

1. Data Ingestion & Matrix Reshaping

R vectors operate in column-major order by default, matching NumPy's order='F' (Fortran indexing) when restructuring data into batch matrices.

# Read raw text data into a vectorvalues <- scan("data.txt", quiet = TRUE)[cite: 1]n <- length(values)[cite: 1]# Simulate normally distributed dataset.seed(42)values_sim <- rnorm(n = 50, mean = 50, sd = 1)[cite: 1]# Reshape vector into a 5x10 matrix (5 samples per hourly batch)# byrow = FALSE preserves Fortran/column-major orderrows <- 5cols <- 10sample_table <- matrix(values, nrow = rows, ncol = cols, byrow = FALSE)[cite: 1]

2. Location & Dispersion Metrics

Base R functions compute sample variance and standard deviation using $n-1$ degrees of freedom by default.

# Base location parametersmean_val   <- mean(values)     # Arithmetic mean x_barmed_val    <- median(values)   # Median x_tildestd_val    <- sd(values)       # Sample standard deviation s (n-1)# Custom harmonic & geometric meanshmean_val  <- 1 / mean(1 / values) # Harmonic meangmean_val  <- exp(mean(log(values))) # Geometric mean# Skewness calculation (requires 'e1071' or custom formula)library(e1071)skew_exact <- e1071::skewness(values, type = 2) # Exact skewnessskew_pearson <- (mean_val - med_val) / std_val # Pearson rule of thumb# Frequency distribution & binning (k = sqrt(n))[cite: 1]k <- ceiling(sqrt(n))[cite: 1]hist_data <- hist(values, breaks = k, plot = FALSE)[cite: 1]

3. Machine Capability Analysis ($C_m$ and $C_{mk}$)

Quantify whether process variation ($6s$) stays within the required operational tolerance limits ($T$)[cite: 1].

calc_capability <- function(data, setpoint = 50, To = 5, Tu = -5) {  s <- sd(data)[cite: 1]  m <- mean(data)[cite: 1]  T_spec <- To - Tu # Total tolerance range[cite: 1]    # Machine capability index Cm[cite: 1]  Cm <- T_spec / (6 * s)[cite: 1]    # Specification limits[cite: 1]  UCL_spec <- setpoint + To[cite: 1]  LCL_spec <- setpoint + Tu[cite: 1]    # Critical deviation and Cmk[cite: 1]  delta_k <- min(UCL_spec - m, m - LCL_spec)[cite: 1]  Cmk <- delta_k / (3 * s)[cite: 1]    return(list(Cm = Cm, Cmk = Cmk, Pass = (Cmk >= 1.67)))[cite: 1]}cap_results <- calc_capability(values)[cite: 1]

4. Gaussian Density Integration & Normal Distribution

Use integrate() to calculate area probabilities under the Gaussian curve $g(x)$[cite: 1].

# Gaussian density function g(x)[cite: 1]g <- function(x, mu = 0, sigma = 1) {  (1 / (sigma * sqrt(2 * pi))) * exp(-0.5 * ((x - mu) / sigma)^2)[cite: 1]}# Integrate over 1s, 2s, and 3s bounds[cite: 1]p_1s <- integrate(g, lower = -1, upper = 1, mu = 0, sigma = 1)$value * 100[cite: 1] # ~68.27%[cite: 1]p_2s <- integrate(g, lower = -2, upper = 2, mu = 0, sigma = 1)$value * 100[cite: 1] # ~95.45%[cite: 1]p_3s <- integrate(g, lower = -3, upper = 3, mu = 0, sigma = 1)$value * 100[cite: 1] # ~99.73%[cite: 1]# Native cumulative distribution equivalentp_3s_native <- (pnorm(3) - pnorm(-3)) * 100[cite: 1]

5. Statistical Process Control (SPC) Quality Control Charts

Compute two-lane control charts using matrix calculations or the native qcc package[cite: 1].

# Method A: Manual Matrix Calculations[cite: 1]A3 <- 1.152; B4 <- 1.669 # Factors for sample size n=5[cite: 1]mw   <- colMeans(sample_table)                  # Hourly batch means[cite: 1]staw <- apply(sample_table, 2, sd)               # Hourly batch std deviations[cite: 1]mmw <- mean(mw)                                  # Mean of means x_double_bar[cite: 1]mws <- mean(staw)                                # Mean of standard deviations s_bar[cite: 1]UCL_m <- mmw + A3 * mws                          # Upper limit (mean chart)[cite: 1]LCL_m <- mmw - A3 * mws                          # Lower limit (mean chart)[cite: 1]UCL_s <- B4 * mws                                # Upper limit (std dev chart)[cite: 1]# Method B: Industrial-grade charts using the 'qcc' package[cite: 1]library(qcc)# Note: qcc expects rows = samples, columns = observations (transpose required)[cite: 1]sample_table_t <- t(sample_table)[cite: 1]par(mfrow = c(2, 1))qcc_xbar <- qcc(sample_table_t, type = "xbar", plot = TRUE) # Mean chart[cite: 1]qcc_s    <- qcc(sample_table_t, type = "S",    plot = TRUE) # Standard deviation chart[cite: 1]

6. Bivariate Linear Regression & Scatter Visualization

Analyze relationships between independent influencing variables ($X$) and dependent outcome variables ($Y$)[cite: 1].

X <- c(46, 53, 29, 61, 36, 39, 47, 49, 52, 38, 55, 32, 57, 54, 44)[cite: 1]Y <- c(12, 15, 7, 17, 10, 11, 11, 12, 14, 9, 16, 8, 18, 14, 12)[cite: 1]# Linear regression model fit y = mx + a[cite: 1]fit <- lm(Y ~ X)[cite: 1]coef_vals <- coef(fit)intercept_a <- coef_vals[1] # Intercept a[cite: 1]slope_m     <- coef_vals[2] # Slope m[cite: 1]# Correlation and Covariance[cite: 1]r_val  <- cor(X, Y)      # Pearson correlation coefficient r[cite: 1]cov_xy <- cov(X, Y)      # Sample covariance S_xy[cite: 1]# Scatter plot with regression line overlay[cite: 1]plot(X, Y, pch = 4, col = "red", xlab = "Relative Humidity (%)", ylab = "Moisture Content")[cite: 1]abline(fit, col = "blue", lwd = 2)[cite: 1]

| Operation | Python (numpy / scipy) | Native R Equivalent |
| --- | --- | --- |
| Reshaping (Column-Order) | np.reshape(arr, (r, c), order='F') [cite: 1] | matrix(arr, nrow=r, ncol=c, byrow=FALSE) [cite: 1] |
| Sample Standard Dev | np.std(arr, ddof=1) [cite: 1] | sd(arr) [cite: 1] |
| Integration | scipy.integrate.quad(func, a, b) [cite: 1] | integrate(func, lower=a, upper=b) [cite: 1] |
| Linear Model | scipy.stats.linregress(X, Y) [cite: 1] | lm(Y ~ X) [cite: 1] |
| Correlation | np.corrcoef(X, Y)[0, 1] [cite: 1] | cor(X, Y) [cite: 1] |

Required R Libraries

| Library | Primary Purpose | Installation |
| --- | --- | --- |
| qcc | Shewhart quality control charts ($\bar{x}$, $s$, $p$, $c$) and capability analysis | install.packages("qcc") |
| e1071 | High-precision skewness, kurtosis, and higher-order distribution metrics | install.packages("e1071") |
| ggplot2 | Statistical graphics, regression scatter overlays, and histogram binning | install.packages("ggplot2") |
| dplyr | Data transformation pipeline (%>%), batch grouping, and aggregation | install.packages("dplyr") |

Data Ingestion & Matrix Structuring

- Load Numeric Stream: v <- scan("data.txt", quiet = TRUE) reads raw line/space-separated numbers directly into a 1D vector.

- Matrix Reshaping: M <- matrix(v, nrow = 5, byrow = FALSE) formats data into batch matrices using column-major ordering.

- Batch Vectorization: colMeans(M) computes mean vector $\bar{x}$; apply(M, 2, sd) calculates batch standard deviation vector $s$.

Location, Dispersion & Distribution Metrics

- Arithmetic Mean ($\bar{x}$): mean(x, na.rm = TRUE)

- Sample Standard Deviation ($s$): sd(x) (uses $n-1$ degrees of freedom)

- Exact Skewness: e1071::skewness(x, type = 2) (sample-size corrected)

- Geometric / Harmonic Means: exp(mean(log(x))) / 1 / mean(1 / x)

- Optimal Binning: k <- ceiling(sqrt(length(x))) | hist(x, breaks = k)

Process Capability Logic ($C_m, C_{mk}$)

R

eval_capability <- function(x, target, tol_upper, tol_lower) {  s <- sd(x); m <- mean(x); T_range <- tol_upper - tol_lower[cite: 1]  Cm <- T_range / (6 * s)[cite: 1]  Cmk <- min((target + tol_upper - m), (m - (target + tol_lower))) / (3 * s)[cite: 1]  list(Cm = Cm, Cmk = Cmk, Pass = (Cmk >= 1.67))[cite: 1]}

Statistical Process Control (qcc Engine)

- $\bar{x}$ Control Chart: qcc(t(M), type = "xbar") constructs the mean control chart using a transposed batch matrix.

- $s$ Control Chart: qcc(t(M), type = "S") generates the process variability chart.

- Capability Visualization: process.capability(qcc(t(M), type = "xbar"), spec.limits = c(45, 55)) plots target capability distribution curves.

Numerical Integration & Normal Probability

- 1D Gaussian Integrator: integrate(function(x) dnorm(x, mean = m, sd = s), lower = a, upper = b)$value

- Native Range Probability: pnorm(b, m, s) - pnorm(a, m, s)

Bivariate Regression & Correlation

- Fit Linear Model: fit <- lm(Y ~ X) fits the line $y = mx + a$

- Extract Slope & Intercept: m <- coef(fit)[2]; a <- coef(fit)[1]

- Correlation & Covariance: cor(X, Y) ($r$); cov(X, Y) ($S_{xy}$)

- Model Diagnostics: summary(fit) extracts $p$-values, $R^2$, and residual standard errors.

Essential R Execution Rules

- Transpose Input Matrices: qcc expects observations as columns and sample batches as rows; always wrap matrices with t(M).

- 1-Based Indexing: R vectors start at index 1, not 0.

- Vectorization First: Avoid explicit for loops; rely on vectorized functions (colMeans(), apply(), pnorm()) for rapid computation.

- Handle Missing Values: Include na.rm = TRUE in aggregation tasks to prevent missing data (NA) propagation.